Celcius 47

# Features

Weight:	2.5 pounds = 1.14 kg

Size:		4 ½ x 4 ½ (115 mm x 115 mm) x 2 ¼” (57 mm) high

Input power:	100-240 VAC 1 amp

UL and CE listed

Temperatures:	Factory calibrated to 47º C  (117º F)

User calibratable from 27º C  (81º F) to 70º C (158º F)

Time to Operating Temperature:  1 minute

Temperature tolerance:  +/- 1º C (2º F)

The Cecius 47 from Horotec is a tool for the purpose of testing watches for leakage after wet pressure testing.  The watch is heated to a temperature of 47º C and a drop of room temperature water is placed on the crystal.   If the watch has leaked condensation will form on the underside of the crystal.

The required precise temperature is obtained through a proprietary solid-state temperature control system that allows for fast heat up and extremely precise temperature control.  The power supply is UL and CE listed and can be used in both 110 volt and 240-volt applications.  The factory-calibrated temperature of 47º C can be changed by the user anywhere between 27º C and 70º C, although 47º C is the perfect temperature not only for testing for condensation but for drying metal watch bands and cases.

Use:  Plug the power supply into an AC source between 100 and 240 volts.  Plug the output of the power supply into the back of the Preciso 47.  Turn on the heater with the switch located on the front left hand side of the panel.  The red LED will glow indicating the unit has power.  The Preciso 47 is factory calibrated to 47 º C (117º F).  The user may change the temperature by inserting a 2 mm watchmaker’s screwdriver in the calibration hole on the bottom of the tester and rotating the trimpot.  If too high a temperature is selected, the red LED power light will cycle on and off.